# Stereoacuity VR Experiment

# Researcher Setup Guide

# Version 1.0

Hardware Required

| Hardware | Purpose |
| --- | --- |
| Windows PC | Runs Unity and MATLAB |
| PICO headset | Presents stereo stimuli |
| PICO controllers | Participant responses |
| Pupil Labs Neon eye tracker | Eye tracking |
| Android Companion Device | Streams Neon data |
| USB-C cable | Companion Device ↔ Neon |
| USB cable | PC ↔ PICO |
| WiFi router | Companion Device ↔ PC communication |

PICO/Pupil labs Gmail account

Name: Steven Dakin

DOB: 01/01/1979

Gender: Male

Address: dakinlabs@gmail.com

Password: P1CO_Integration2026

Recovery email address: dralicecade@gmail.com

Recovery ph: (Alice Cade): 021400739

Phone pin: 2026

Pupil cloud details

Username: dakinlabs

Address: dakinlabs@gmail.com

Password: P1CO_Integration2026

Experiment Setup

Network Layout

WiFi Router

(same subnet)

PC -------------- Android Companion Device

|                        |

| USB-C             | USB-C

|                        |

PICO                    Neon

The important point is:

PICO does not communicate with Neon.

Unity communicates directly with the Android Companion Device.

The Companion Device communicates with the Neon over USB.

Before Beginning, ensure

□ Companion Device charged

□ PICO charged

□ Neon connected to Companion Device

□ Companion Device connected to WiFi

□ PC connected to the SAME WiFi

□ PICO connected to PC via USB

Step 1

Connect Hardware

PICO

Connect PC > USB-C > PICO

Do not unplug this cable.

NeonConnect

Companion Device >  USB-C >  Neon

White LED should illuminate.

Participant Setup

Fit: PICO headset & PICO controllers

Step 2

Open Companion App. Launch Neon Companion

Confirm

✓ Workspace shown

✓ Wearer selected

✓ No errors

Step 4

Find Companion Device IP on Android

Settings >WiFi> Current Network> Advanced>IP Address

Example

172.24.14.13

This IP changes between networks. Always check it before each experiment.

Step 5

Start Unity

Open > StereoacuityLSLUnity > Press Play

Researcher setup panel appears.

Enter IP address companion device (i.e., 172.24.14.13)

Press > Save IP. Status should indicate the IP has been saved.

Press> Start Experiment

Step 6

Confirm Eye Tracking. Unity Console should show eye tracking data:

Gaze:

origin=

direction=

point=

If gaze values continually update, the connection is successful.

Step 7

Start MATLAB

Navigate to: Stereoacuity_Unity > MATLAB

Run: MakeStereoStimulus.m

MATLAB Startup Inputs

The experiment currently asks for:

Participant ID: ie 001

Number of QUEST trials? [20]: add number wanted

Initial log threshold guess? [-1]: n input -1

Guess SD? [2]: input 2

MATLAB Connection

MATLAB should report “Connected to Unity”

Unity should report “TCP Client Connected”

Step 8

Calibration

(Currently) Neon calibration is automatic.

No manual calibration is required.

(If we later add a calibration scene, it would go here.)

Step 9

Begin Experiment

Press: Space

Participant completes QUEST staircase.

Responses return to MATLAB.

Expected Data

Unity

timestamps

gaze vectors  (TBC)

gaze point (TBC)

MATLAB

QUEST threshold

trial responses

stimulus presentation timing

Troubleshooting

Unity says: No device discovered

Usually = wrong IP

Unity says: Started playing session, but no gaze

Check if Neon connected with White LED

MATLAB cannot connect

Check

Unity already in Play mode

Port

5005

not blocked

PICO blank screen

Check if USB connected

SteamVR/OpenXR running

Unity Play mode

May need to restart PICO

Daily Researcher Checklist

□ Neon connected

□ Companion Device connected

□ Companion Device streaming

□ Companion IP entered

□ Unity connected

□ Gaze data visible

□ MATLAB connected

□ Participant comfortable

□ Recording started